Window Shade Control

Introduction

This group of PGN’s applies to devices that drive and control DC powered motorized window shades. The following formats apply (see Table 1).

Table 1 – Window Shade Control Definition

| Device attribute | Value |
| --- | --- |
| Category | Power components |
| Default Source Address | 131 |
| Dynamic Address Range | 128-143 |
| Instance | Multiple |

Static addressing is discouraged in this product.

Window Shade Control Status

The following table (Table 2) defines the PG attributes and Table 3 defines the signal and parameter attributes.

Table 2 – PG Definition

| PG attribute | Value |
| --- | --- |
| Name | WINDOW_SHADE_CONTROL_STATUS |
| PGN | 1FEDEh (pending) |
| Default priority | 6 |
| Maximum broadcast rate | N/A |
| Normal broadcast rate | on change |
| Minimum broadcast rate | N/A |
| Number of frames | 1 |
| ACK requirements | None |

Table 3 – Signal and parameter definition

| Byte | Bit | Name | Data type | Unit | Value description |
| --- | --- | --- | --- | --- | --- |
| 0 | - | Instance | uint8 | - | Valid = 1 to 250. |
| 1 | - | Group | uint8 | bitmap | Indicates group membership. One bit is assigned to each of seven groups. Highest bit is not used to indicate a group. If 0, t he other bit s refer to the groups. 0 in any other position indicates the load is a member of the corresponding group. 01111110 – Group 1 01111101 – Group 2 00000000 - All groups 11111111 – No Data |
| 2 | - | Operating Status (Motor Duty) | uint8 | Percent | 0-200 = 0-100% in 0.5% increments. See table 4 from ISO_21zzz-7_Draft2_E_.pdf. |
| 3 | 0 to 1 | Lock Status | Bit2 | - | 00b – Load is unlocked 01b – Load is locked 11b – Lock command is not supported When locked, the instance will ignore certain commands until an unlock command is received. |
|  | 2 to 3 | Motor Status | Bit2 | - | 00b — Neither ‘Forward’ nor ‘Reverse’ output is on. 01b — either ‘Forward’ or ‘Reverse’ output is on (Motor active in either direction) |
|  | 4 to 5 | Forward Status | Bit2 | - | 00b — ‘Forward’ output not on 01b — ‘Forward’ output is on |
|  | 6 to 7 | Reverse Status | Bit2 | - | 00b — ‘Reverse’ output not on 01b — ‘Reverse’ output is on |
| 4 | - | Duration | uint8 | Sec | Number of seconds remaining in Duration command. Max = 240 seconds 0 = delay/duration expired 1 – 239 = seconds remaining 240 = 240 or more seconds remaining 255 = no delay/duration active |
| 5 | - | Last Command | uint8 |  | Indicates last command (function) executed by this instance. This is the last command executed by the WINDOW_SHADE_CONTROL_COMMAND (PGN 1FEDF). See Table 4 below for a list of possible commands. |
| 6 | 0 to 1 | Overcurrent Status | Bit2 | - | 00b — load output not in overcurrent 01b — load output has drawn overcurrent 11b – Overcurrent status is unavailable or not supported |
|  | 2 to 3 | Override Status | Bit2 | - | 00b – External override is inactive 01b – External override is active 11b – Override status is unavailable or not supported When the override is active, the output has been physically changed by a user outside of the device. |
|  | 4 to 5 | Disable1 Status | Bit2 | - | 00b — Disable 1 is not active 01b — Disable 1 is active 11b – Disable 1 is not supported When disable 1 is active, it has been set through an external signal input. |
|  | 6 to 7 | Disable2 Status | Bit2 | - | 00b — Disable 2 is not active 01b — Disable 2 is active 11b – Disable 2 is not supported When disable 2 is active, it has been set through an external signal input. |
| 7 | - | Reserved |  |  |  |

Table 4 – Commands Possible for “Last Command”

| Command | Description |
| --- | --- |
| 0x04 – Stop | If motor is active in either direction, immediately stop it. |
| 0x81 – Forward | Turn motor on in the “Forward” (open shade) direction for specified duration (Controller may enforce a dead time for direction reversal) |
| 0x41 – Reverse | Turn motor on in the “Reverse” (close shade) direction for specified duration (Controller may enforce a dead time for direction reversal) |
| 0x85 - Toggle Forward | If motor is off, turn on in the forward direction for specified duration. If motor is on in forward direction, stop it. If motor is on in reverse direction, stop it and turn on in forward direction (Controller may enforce a dead time for direction reversal). |
| 0x45 - Toggle Reverse | If motor is off, turn on in the reverse direction for specified duration. If motor is on in reverse direction, stop it. If motor is on in forward direction, stop it and turn on in reverse direction (Controller may enforce a dead time for direction reversal). |
| 0x10 – Tilt | Tilts louvers type shades in the upward direction. |
| 0x21 – Lock | Lock instance preventing certain commands from affecting it until an “Unlock” command is received. |
| 0x22 – Unlock | Unlock an instance that is currently locked. |

Window Shade Control Command

The following table (Table 5) defines the PG attributes and Table 6 defines the signal and parameter attributes.

Table 5 – PG Definition

| PG attribute | Value |
| --- | --- |
| Name | WINDOW_SHADE_CONTROL_COMMAND |
| PGN | 1FEDFh (pending) |
| Default priority | 6 |
| Maximum broadcast rate | N/A |
| Normal broadcast rate | as needed |
| Minimum broadcast rate | N/A |
| Number of frames | 1 |
| ACK requirements | ACK, WINDOW_SHADE_CONTROL_STATUS |

Table 6 – Signal and parameter definition

| Byte | Bit | Name | Data type | Unit | Value description |
| --- | --- | --- | --- | --- | --- |
| 0 | - | Instance | uint8 | - | Valid = 1 to 250. 0xFF for group commands. |
| 1 | - | Group | uint8 | bitmap | see Table 203 [external doc reference] If bit 7 = 1 and bit 6 = 0, it is a node group. Node groups support more then seven groups where multiple groups cannot be selected in one command. This is required to handle the physical grouping of multiple control instances within one node. 10000001 – Node Group 1 10011111 – Node Group 31 (max for this mode) 11111111 – For non-group commands |
| 2 | - | Operating Status (Motor Duty) | uint8 | % | 0-200 = 0-100% in 0.5% increments |
| 3 | - | Command | uint8 | Bitmap | 0x04 – Stop 0x81 – Forward 0x41 – Reverse 0x85 – Toggle Forward 0x45 – Toggle Reverse 0x10 – Tilt 0x21 – Lock 0x22 – Unlock See table 4 above for an explanation of each command. If a node does not support some of the above commands, it must return a NACK – Command Not Supported. |
| 4 | - | Duration | uint8 | Sec | Number of seconds to enable motor for before stopping. Range: 1 to 240 seconds Additional minute increment values: 241 = 5 min 242 = 6 min … 250 = 14 min 255 = Continuous command 0 = momentary command. This command will only turn on the channel for 2 seconds. However, if the message is repeated with a period less than 2 seconds, the channel will stay on indefinitely. Duration is ignored if not supported by specified command |
| 5 | - | Interlock | uint8 | Bitmap | Bit 0 – Interlock A Bit 1 – Interlock B A command message with either interlock bit set to ‘1’ will not be activated until an identical message is received from a different source with the opposing interlock bit set to ‘1’. Bit 2-7 – Reserved |
| 6-7 | - | Reserved |  |  |  |